Makor Oven System

Substrate Specifications

Substrates having the following approximate maximum and minimum dimensions can be transported through the equipment offered:

| Length | maximum | 6500 mm | (256") |
| --- | --- | --- | --- |
|  | minimum | 305 mm | (12") |
| Width | maximum | 1300 mm | (51") |
|  | minimum | 50 mm | (2") |
| Thickness | maximum | 50 mm | (2") |
|  | minimum | 10 mm | (3/8") |

Scope Of Delivery

- Timed Belt Conveyor, Makor Model ROLLY 3500/T

- Multilevel Oven System, Makor Model MULTILEVEL 7000/6

Item 1

Timed Belt Conveyor

Makor Model ROLLY 3500/T

Timed Belt Conveyor

belt conveyor to use as an infeed conveyor for the system

utilized for the manual loading process

designed to create groups or charges for loading into multilevel oven

transport system is located inside a solid steel structure

synthetic transport belt is resistant to abrasion and chemical agents

forward feed is adjustable by inverter

Options Included

equipped for timed/charge formation

one (1) independent control panel mounted on conveyor

Technical Specifications

| overall dimensions | 3500 x 1600 x 900 mm | (138 x 63 x 35-1/2") |
| --- | --- | --- |
| max. working width | 1300 mm | (51") |
| working height ± 50 mm | 900 mm | (35-1/2") |
| feed speed ± 5% | 2 to 10 m/min | (6.6 to 32.8 ft/min) |

Utility Requirements

| Electrical |  |  |
| --- | --- | --- |
| operating voltage | 480 V/3-phase/60 Hz |  |
| total connected load | 0.55 kW |  |

Electrical equipment installed according to UL regulations.  Voltage supplied must not fluctuate in excess of +/- 5% of its stated value.

Item 2

Multilevel Oven System

Makor Model MULTILEVEL 7000/6

The MULTILEVEL oven allows the flash off of water-based or solvent-based coating materials.  Each tray has its own independent transport belt to assure the best cleaning of the flash off zones.  This is extremely important to achieve a perfect finishing quality.

The PLC manages the machine, and allows selecting the number of stages with which the operator wants to work to achieve maximum flexibility in choosing the drying time.

Structure

the structure is made of sturdy painted steel profiles and is completely insulated with panels, with sliding doors for entry into the machine

inside the insulated chamber are located the belt stages to store the pieces

all of the trays are movable vertically and horizontally and can be positioned, alternatively and independently, to the line working stage

the geared transmission unit automatically synchronizes with the line transport system to load pieces coming from the previous machines and then off-load pieces that have already been flashed off

vibration-free vertical movement of the trays is achieved using a screw system

the value control system provides perfect positioning of the stage column

Air Flow Management

flash off zone is equipped with one fan for intake of hot air and an exhaust fan that removes the vapors released during the flash off phase

recycling filters are located on the air intake system

the air is heated by steam

Options Included

automatic temperature control

a sensor detects the air temperature inside the oven and automatically optimizes the heat exchanger that warms up the air

this system allows using the maximum potential of the heating system when the machine is started up and saving energy once the set temperature is reached

prepared for operation with steam

equipped with two-meter outfeed roller element to assist with the manual unloading process

Technical Specifications

| overall dimensions | 8700x3000x2650mm | (343 x 118 x 104") |
| --- | --- | --- |
| max. working dimensions | 6500x1300x 50mm | (255 x 51 x 2") |
| feed speed | 2 to 10 m/min | (6.6 to 32.8 ft/min) |
| working height ±50 mm | 900 mm | (35-1/2") |
| number of trays | 6 |  |
| tray length | 7000 mm | (275-1/2") |
| maximum load each tray | 120 kg | (265 lb) |
| feed motor power | 0.55 kW |  |
| roller conveyor motor power x2 | 0.25 kW each |  |
| tray motor power | 4 kW |  |
| intake motor power | 3 kW |  |
| exhaust motor power | 3 kW |  |
| control side | right hand |  |

Utility Requirements

| Electrical |  |  |
| --- | --- | --- |
| operating voltage | 480 V/3-phase/60 Hz |  |
| total connected load | 12.0 kW |  |
| Air |  |  |
| exhaust air volume | 6,000 m3/hr | (3,532 cfm) |
| air make up required | 6,000 m3/hr | (3,532 cfm) |
| Heat Source |  |  |
| thermal power | 40,000 kcal/h | (160,000 BTU) |

Electrical equipment installed according to UL regulations.  Voltage supplied must not fluctuate in excess of +/- 5% of its stated value.